Maxidecoder v1.0

Introduction:

Maxidecoder v1.0 is a very cheap ( about 15 US$ ) easy to build decoder for cable TV networks using syncronisation coding.

The onely limit to this decoder is that you need a device ( TV or Video ) with a scart interface to use this decoder.

I’ve tested it at the Swedish network ”Kabelvisionen” with great results. It should allsow work at ”Telia” and ”Svenska Kabel TV”.

It manage to decode all 28 channels ”Kabelvisionen” offering including Filmnet 1 & 2,

TV1000, TV1000 Cinema and Hallmark.

Please send any coment or if you’ve tested it on another system and it works to

nicron@bigfoot.ocm

Function description:

There are many was to code a TV signal. Some companies uses a signal that scrambled the TV signal and are removed with a ”mid pass” filter. Such a picture are recognized  by it’s stable but not vary nice to look at. Maxidecoder v1.0 doesn’t decode this kind of signal.

An other way to code a TV signal is to manipulate with the signals syncronisation and polarity. The coded picture may roll and you can’t se a thing. You can’t hear anything eigther. When the picture ain’t syncronised, the TV wont know where the picture starts or ends. Maxidecoder v1.0 is made for this.

Components:

IC 1	1	Logical unit. 4069 ( 14 legs )

IC2	1	Logical unit. 4538 ( 16 legs )

p1, p3	2	Potentiometer 22kW or 25kW

p2	1	Potentiometer 10kW

p4	1	Potentiometer 100kW

p5	1	Potentiometer 2,2M or 2,5MW

p6	1	Potentiometer 100W

r1,r6	2	Resistor 100kW

r2	1	Resistor 10kW

r3	1	Resistor 56kW

r4	1	Resistor 47kW

r5	1	Resistor 18kW

r7	1	Resistor 6,2kW

r8	1	Resistor 22kW

r9	1	Resistor 47W

c1	1	Ajustable Capasitor 15p

c2	1	Capasitor 1000 pf

c3, c4, c7	3	Capasitor100pf

c5, c8	2	Capasitor 2200 pf

c6	1	Capasitor 68 pf

d1, d2, d3, d4	4	Diod 4148

Scart connector	1

RF contacts	2	Male, Female

How to build it:

1.	Solder the two IC’s.

2.	Solder all potentiometers.

3.	Solder all capasitors.

4.	Solder all resistors.

5.	Solder all diods.

6.	Connect a 6 -12V DC supply or a 9V batteri connector.

7.	Attach the scart connector.

8.	Solder the koax cable and the RF connectors.

9.	Jump high and hope you’ve got all at the right palce.

How to connect it:

This decoder may be connected to a TV or Video that are equipped with a scart connector.

Make sure that you connect the koax and the scart connector to the same device. It wont work unless. Connect the antenna to the decoder.

Test and adjustment:

Note:

Don’t proceed with the next step until the recent one is finnished.

1. Power on the decoder. Switch to a UNCODED channel. Adjust the TV to normal brigthness. Make the following adjustments:

p1 fully to the right, p2 centerd, p3 centerd, p4 fully to the right, p5 fully to th left, p5 fully to the right and c1 centerd.

2. When you are finnished with this step, you’ll have a brigth wide line over the screen.

Turn p2 until you have a stable picture. If  you can’t get it stable, try adjusting p3 a bit and then p2 again.

3. When you are finnished with this step, the wide line will be gone but the picture is brigther than before.

Turn p4 until the wide line just dissapear to the left. If you can’t see the left edge, turn p4 fully left and then turn it back until it dissapers.

Turn p1 until the rigth edge dissapears to the right.

4. Adjust p5 until the distortions in the top of the sceen just dissapears.

5. Switch to an CODED channel. Turn p6 fully left. Adjust c1 until the coded channel appears. Note that the picture may dissapear when you touch c1 with a iron screwdriver.

Turn to another coded channel and adjust c1 until it appears.

Now you have to get true all coded channels and find the right setting for c1.

When this is done, you can adjust the coded channels brigthness with p6. Here you will have to compromise besauce you can’t get the same brightness at all channels, the decoder ain’t that good yet.

When it’s not working:

- The picture has totaly disapeard.

If you heaven’t got any picture at any coded channel it’s probably p2 that needs an adjustment.

If this ain’t working, try increase the brigthness with p6 and if this ain’t working try c1.

- The picture is unstable at the top.

Try p2 and if that ain’t working at all try p5. If nothing of this works, try turn p3 a little bit more to the right.

- The picture is unstable in the sides.

Turn up p6. if this doesn’t work, try p4 or p3.

- Prolems when connecting it to another TV or Video.

Adjust p3 for the incoming signal level, and p6 / c1 for the brightness.

- The picture is to bright.

Is it an uncoded cannel ? If it is, switch off the decoder and if not, adjust p6 / c1.

- I can’t get the autosearch to stop at a coded channel.

Try adjust p6 / c1 for maximum brightness. It will find the cannels easier then.

//NiCRoN of T.S.Y.S  ( The Swedish Yankees )

nicron@bigfoot.com

65 mm.

100 mm.