Description IHA 30

The HF-gencrator IHA 30 has a rated output of max. 3 kW.

The generator is enclosed in a aluminiumeabinet.  Two sides of the generator are fitted with removable panels.

Controls, instruments, HF-terininal and signal lamps are placed on the front of the generator, the mainsand water-connections on the opposite side.

The generator is equipped for soldering of carbide tips on circular sawblades.  The generator can be fitted with, external footpedal for start and stop of HF-power and adjustable transfonner for power regulation.

Technical data: Electrical connection: as specified 22OV or I lOV 1 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 5 kVA.  Frequency: ca 1,5 MHz.

Oscillator tube: YD 11 50

Rectifiers:	4 pcs.  ED 5417

Weight: ca 90 kg.

Water consumpfion: 0,5-1 I/min.

1.	Output terminal.

2.	Switch mains manual-timer

3. Timer

4.	Stop I-IF.

5.	Start IIF.

6. Lamp EF.

7.	Lamp ready.

8.	Anode current.