Description IHA 800

The HF-generator IHA 800 has a rated output of max. 80 kW.

The generator is enclosed in a aluminiumcabinet.  Two sides of the generator are fitted with removable panels.

Controls, instruments, HF-terminal and signal lamps are placed on the front of the generator, the mainsand water-connections on the opposite side.  The output power is continuous regulated from I 0 to 100%.

The capacitor-battery contains six water cooled capacitors, reversible to be adjusted to heat treatment of aswell magnetic as um-nagnetic material.

The generator is equipped with a over load circuit breaker to avoid overloading and its also equipped with a time relay for the HF-time.

Technical data: Electrical connection: as specified 220-50OV 3 phase, 5OHZ or 60Hz.

Power consumption at maximum output power: 160 kVA.

Frequency:	app. 300 kHz, Oscillator tube: YD 1202, Rectifiers: Silicon diodes.

Weight:	app. 1300 kg, Water consumption: app. 30 I/min.

1080             180         1385

1.	Power control, 2. Anode current.

3.	Grid current, 4. High voltage.

5.	Lamp control voltage.

6.	Lwnp ready, 7. Lamp HF.

8.	Stop HF, 9. Start HF.

I	0. Waterflow indicator

I	1. Watertemp indicator.

12.	Overload circuit breaker indicator

13.	Switch timerelay-man.

14.	Timerelay.

15.	Power output.

16.	Gridcontrol.

17.	Highvoltage switch.

18.	Overload circuit breaker.

19.	Mains connection.

20.	Water connection.