Cool'ing systein IKS 88.0100

Description

The clo sed cooling system KS 8 8. 0 1 00 is made for cooling of HF-gencrators and sirnilar equipment where a clean cooling water and controlled temperature is required.

Technical data: The unit is built on a welded construction and consists of.

Pump type NMD 20/140B, wilh a capacity of 16/80 Fmin. at 5,3/3, 5 bar, motor 1, 1 kW 2900 r/m. Tank of stainless steel containing 30 litres. Heat exchanger, type Ultra Flex UFX-6 C with 12 cooling plates of stainless steel.

Thermostat, type AVTA 20, adjustable between 25-65 degrees C.

All components having contact with water are made of non-rusting material such as copper, brass and stainless steel.

Weight:	app. 100 kg.

1. Tank

2. Pump

3.	Thermostat

4.	Motor protector

5.	Heat exchanger