The Linux CD-ROM HOWTO

Jeff Tranter, jeff_tranter@pobox.com

v1.10, 5 January 1997

This document describes how to install, configure, and use CD-ROM

drives under Linux. It lists the supported hardware and answers a num

ber of frequently asked questions. The intent is to bring new users up

to speed quickly and reduce the amount of traffic in the Usenet news

groups and mailing lists.

1.  Introduction

This is the Linux CD-ROM HOWTO. It is intended as a quick reference

covering everything you need to know to install and configure CD-ROM

hardware under Linux. Frequently asked questions related to CD-ROM are

answered, and references are given to other sources of information

related to CD-ROM applications and technology.

1.1.  Acknowledgments

Much of this information came from the documentation and source files

provided with the Linux kernel, the Internet alt.cdrom newsgroup FAQ,

and input from Linux users.

Thanks to the Linuxdoc-SGML package, this HOWTO is available in

several formats, all generated from a common source file.

1.2.  Revision History

Version 1.0

First version made publicly available

Version 1.1

CDU33A is explicitly supported as of 1.1.20 kernel; notes on

Reveal FX; info on reading audio tracks; info on some alpha

drivers; added troubleshooting section; a few other minor

additions

Version 1.2

ISO-9660 file systems must be mounted read-only starting with

1.1.33 kernel; clarified that SB16 SCSI is supported and newer

Aztech drives are not supported; references to photocd and xpcd

programs; note on new SBPCD auto eject feature

Version 1.3

Minor change to the way SBPCD eject feature is disabled starting

with the 1.1.49 kernel; added info on XA discs and how to

identify them

Version 1.4

HOWTO now available in other languages; IBM and Longshine drives

now supported by SBPCD; alpha driver for Aztech drives; CDU-33

driver no longer auto-probes, supports PhotoCD and audio; more

than 2 SCSI drives are supported; new driver for IDE; reminder

to check drive jumpers; can now set SBPCD auto-eject with IOCTL;

list drivers with multisession support; question on flashing

light on CDU-33

Version 1.5

A long overdue update (I've been busy); document placed under

GPL; info on many new kernel drivers; more info on configuration

and troubleshooting; lots of HTML links added; many other minor

changes

Version 1.6

Added link to eject program; question on file permission patch;

link to Creative Labs Web site; reference to ATA/EIDE FAQ and

FTP site; note that many Creative Labs and Mitsumi drives are

now EIDE; mention Supermount; drives listed as supporting

digital data are obsolete, refer to cdda2wav; more info on

writing CDs; multi-disc EIDE drive info; a few typos fixed

Version 1.7

new kernel version; most README files moved to

/usr/src/linux/Documentation; some drivers are no longer

experimental; a few more supported drives; emphasize that most

drives are now IDE/ATAPI; added questions on Plug and Play

support and identifying drive speed; vger mailing lists being

shut down; other miscellaneous minor changes

Version 1.8

question on why CD-ROM stops working after install; aztcd driver

now supports two CyCDROM drives; more pleading not to use SBPCD

driver with IDE drives; some ATAPI multi-disc changer support;

note on (lack of) support for parallel port drives; latest

stable kernel is 2.0; other miscellaneous minor changes

Version 1.9

removed some questions that were very old and now obsolete; new

e-mail address for author; Chinese translation available; fixed

some links to point to latest software packages; more

information on multimedia book; minor spelling and grammatical

changes

Version 1.10

clarify that new Aztech drives are ATAPI; added CDU31A options

for modular driver; mount option for reading hidden files; added

six month "best before" date; new URL to web page for book;

minor spelling and grammatical changes

1.3.  New Versions Of This Document

New versions of this document will be periodically posted to the

comp.os.linux.answers newsgroup. They will also be uploaded to various

anonymous ftp sites that archive such information including

<ftp://sunsite.unc.edu/pub/Linux/docs/HOWTO/>.

Hypertext versions of this and other Linux HOWTOs are available on

many World-Wide-Web sites, including

<http://sunsite.unc.edu/mdw/mdw.html>. Most Linux CD-ROM distributions

include the HOWTOs, often under the  </usr/doc/> directory, and you

can also buy printed copies from several vendors. Sometimes the HOWTOs

available from CD-ROM vendors, ftp sites, and printed format are out

of date. If the date on this HOWTO is more than six months in the

past, then a newer copy is probably available on the Internet.

A French translation of this document, by Bruno Cornec

(cornec@stna7.stna.dgac.fr) is available at

<ftp://ftp.ibp.fr/pub2/linux/french/docs/HOWTO/>.

A Japanese translation by Itsushi Minoura (minoura@uni.zool.s.u-

tokyo.ac.jp) is available from  <http://jf.linux.or.jp/JF/JF.html/>.

A Chinese translation (BIG-5 encoding) by Yung-kang Wu

(yorkwu@in3.csie.ncu.edu.tw) is available from

<http://www.csie.ncu.edu.tw/~yorkwu/linux/howto/cdrom/>.

If you make a translation of this document into another language, let

me know and I'll include a reference to it here.

1.4.  Feedback

I rely on you, the reader, to make this HOWTO useful. If you have any

suggestions, corrections, or comments, please send them to me,

jeff_tranter@pobox.com, and I will try to incorporate them in the next

revision.

I am also willing to answer general questions on CD-ROM and Linux, as

best I can. Before doing so, please read all of the information in

this HOWTO, and send me detailed information about the problem.

If you publish this document on a CD-ROM or in hardcopy form, a

complimentary copy would be appreciated; mail me for my postal

address. Also consider making a donation to the Linux Documentation

Project to help support free documentation for Linux. Contact the

Linux HOWTO coordinator, Greg Hankins (gregh@sunsite.unc.edu), for

more information.

1.5.  Distribution Policy

Copyright 1995-1997 Jeff Tranter.

This HOWTO is free documentation; you can redistribute it and/or

modify it under the terms of the GNU General Public License as

published by the Free Software Foundation; either version 2 of the

License, or (at your option) any later version.

This document is distributed in the hope that it will be useful, but

without any warranty; without even the implied warranty of

merchantability or fitness for a particular purpose.  See the GNU

General Public License for more details.

You can obtain a copy of the GNU General Public License by writing to

the Free Software Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139,

USA.

2.  CD-ROM Technology

"CD-ROM is read-only memory, and audio compact disc system is

available as package-media of digital data for those purpose. For

playing audio CD, please insert Head-phone jack."

--- from a CD-ROM instruction manual

Don't Panic! The world of CD-ROM technology is not as confusing as

your instruction manual.

CD-ROM stands for Compact Disc Read-Only Memory, a mass storage medium

utilizing an optical laser to read microscopic pits on the aluminized

layer of a polycarbonate disc. The same format is used for audio

Compact Discs. Because of its high storage capacity, reliability, and

low cost, CD-ROM has become an increasingly popular storage media.

The storage capacity of a CD-ROM disc is approximately 650 megabytes,

equivalent to over 500 high density 3.5" floppy disks or roughly

250,000 typed pages.

First generation drives (known as single speed), provided a transfer

rate of approximately 150 kilobytes per second; these are now

generally considered obsolete. Hardware manufacturers have introduced

double speed (300 kB/sec), quad speed (600 kB/sec), 6 times (6X), 8

times (8x), and even higher speed drives.

Most CD-ROM drives use either the Small Computer Systems Interface

(SCSI), ATAPI enhanced IDE interface, or a vendor proprietary

interface. They also typically support playing audio CDs via an

external headphone jack or line level output. Some CDs also allow

reading the frames of data from audio CDs in digital form.

CD-ROMs are usually formatted with an ISO-9660 (formerly called High

Sierra) file system. This format restricts filenames to the MS-DOS

style (8+3 characters). The Rock Ridge Extensions use undefined fields

in the ISO-9660 standard to support longer filenames and additional

Unix style information (e.g. file ownership, symbolic links, etc.).

PhotoCD is a standard developed by Kodak for storing photographic

images as digital data on a CD-ROM. With appropriate software, you can

view the images on a computer, manipulate them, or send them to a

printer. Information can be added to a PhotoCD at a later date; this

is known as multi-session capability.

CD recorders (CD-R) have recently become available. They use a

different media and specialized equipment for recording, but the

resulting disc can be read by any CD-ROM drive.

In the future CD-ROM drive vendors are expected to offer new

technologies that will increase storage capacity by an order of

magnitude.

3.  Supported Hardware

This section lists the CD-ROM drivers and interfaces that are

currently supported under Linux. The information here is based on the

latest stable Linux kernel, which at time of writing was version

2.0.27. A development kernel (2.1.x versions) is also available but is

not guaranteed to be stable.

This information is only valid for Linux on the Intel platform. Much

of it should be applicable to Linux on other processor architectures,

but I have no first hand experience or information.

3.1.  ATAPI CD-ROM Drives

ATAPI (ATA Packet Interface) is a protocol for controlling mass

storage devices. It builds on the ATA (AT Attachment) interface, the

official ANSI standard name for the IDE interface developed for hard

disk drives. ATAPI is commonly used for hard disks, CD-ROM drives,

tape drives, and other devices.  Currently the most popular type of

interface, it offers most of the functionality of SCSI, without the

need for an expensive controller or cables.

The Linux kernel has a device driver that should work with any ATAPI

compliant CD-ROM drive. Vendors shipping compatible drives include

Aztech, Mitsumi, NEC, Sony, Creative Labs, and Vertos. If you have

recently purchased a CD-ROM drive, especially if it is quad speed or

faster, it is almost guaranteed to be IDE/ATAPI.

3.2.  SCSI CD-ROM Drives

SCSI (Small Computer Systems Interface) is a popular format for CD-ROM

drives. Its chief advantages are a reasonably fast transfer rate,

multi-device capability, and support on a variety of computer

platforms. Some disadvantages of SCSI are the need for a relatively

expensive controller card and cables.

Any SCSI CD-ROM drive with a block size of 512 or 2048 bytes should

work under Linux; this includes the vast majority of CD-ROM drives on

the market.

You will also need a supported SCSI controller card; see the SCSI

HOWTO for more information on interface hardware.

Note that some CD-ROMs include a proprietary controller with a

modified interface that is not fully SCSI compatible (e.g. it may not

support adding other SCSI devices on the bus). These will most likely

not work under Linux.

3.3.  Proprietary CD-ROM Drives

Several CD-ROM drives using proprietary interfaces are available; the

interface is often provided on a sound card. Simple interface cards

equivalent to that provided on the sound card are also available.

These drives generally tend to be lower in cost and smaller than SCSI

drives. Their disadvantages are the lack of standardization and

expandability.

Note that proprietary interfaces are sometimes erroneously referred to

as IDE interfaces, because like IDE hard disks, they use a simple

interface based on the PC/AT bus. To add to the confusion, some

vendors, most notably Creative Labs, have shipped many different types

of CD-ROM drives and have offered proprietary, SCSI, and ATAPI

interfaces on their sound cards.

The table below lists the proprietary CD-ROM drives that are known to

be supported under Linux. Drivers for additional devices may be

available in the latest development kernels or as kernel patches. The

latter can most often be found at

<ftp://sunsite.unc.edu/pub/Linux/kernel/patches/cdrom/>. Also check

the README files included with the kernel distribution, usually

installed /usr/src/linux/Documentation/cdrom/, for the latest

information.

Proprietary CD-ROM Drives

Vendor          Model           Kernel Driver   Notes

------          -----           -------------   --------

Panasonic       CR-521          sbpcd           Note 1

Panasonic       CR-522          sbpcd           Note 1

Panasonic       CR-523          sbpcd           Note 1

Panasonic       CR-562          sbpcd           Note 1

Panasonic       CR-563          sbpcd           Note 1

Creative Labs   CD-200          sbpcd

IBM             External ISA    sbpcd           Note 2

Longshine       LCS-7260        sbpcd

Teac            CD-55A          sbpcd

Sony            CDU-31A         cdu31a

Sony            CDU-33A         cdu31a

Sony            CDU-535         sonycd535       Note 3

Sony            CDU-531         sonycd535

Aztech          CDA268-01A      aztcd           Note 4

Orchid          CDS-3110        aztcd

Okano/Wearnes   CDD110          aztcd

Conrad          TXC             aztcd

CyCDROM         CR520ie         aztcd

CyCDROM         CR940ie         aztcd

GoldStar        R420            gscd            Note 5

Philips/LMS     CM206           cm206           Note 6

Mitsumi         CRMC LU005S     mcd/mcdx        Note 7, 8

Mitsumi         FX001           mcd/mcdx        Note 7, 8

Optics Storage  Dolphin 8000AT  optcd

Lasermate       CR328A          optcd

Sanyo           H94A            sjcd

various         various         isp16           Note 9

Notes:

1. These drives may be sold under the names Creative Labs, Panasonic,

Matsushita, or Kotobuki.

2. This drive is the same as a Panasonic CR-562.

3. May also be sold under the Procomm name.

4. This driver is for the CDA268-01A only. Other models, including the

CDA268-03I and CDA269-031SE are not proprietary and should use the

IDECD (ATAPI) kernel driver.

5. May also be sold as part of a Reveal Multimedia Kit.

6. The Philips CM205 is not supported by this driver, but there is a

separate alpha release driver available from ftp://sunsite.unc.edu

in /pub/Linux/kernel/patches/cdrom/lmscd0.4.tar.gz

7. May also be sold under the Radio Shack name.

8. There are two drivers available. "mcd" is the original one, and

"mcdx" is a newer driver with more features (but possibly less

stable).

9. This driver works with CD-ROM drives that are attached to the

interface on an ISP16, MAD16 or Mozart sound card.

If a drive listed here is not supported by your kernel, you probably

need to upgrade to a newer version.

If your drive is not one of the models listed here, particularly if it

was bought recently and is quad speed or faster, it probably uses the

IDE/ATAPI interface listed in a previous section. The single most

common error among Linux CD-ROM users is to assume that any drive

connected to a SoundBlaster card should use the SBPCD driver. Creative

Labs and most other vendors are no longer selling proprietary

interface drives, they are following the standard ATAPI/IDE interface.

3.4.  Parallel Port Drives

Some vendors sell CD-ROM drives that attach to the PC parallel port.

The standard Linux kernel does not support these drives, and I am not

aware of anyone working on drivers.

4.  Installation

Installation of a CD-ROM under Linux consists of these steps:

1. Installing the hardware.

2. Configuring and building the Linux kernel.

3. Creating device files and setting boot time parameters

4. Booting the Linux kernel.

5. Mounting the media.

The next sections will cover each of these steps in detail.

4.1.  Installing the Hardware

Follow the manufacturer's instructions for installing the hardware or

have your dealer perform the installation. The details will vary

depending on whether the drive is internal or external and on the type

of interface used. There are no special installation requirements for

Linux. You may need to set jumpers on the drive and/or interface card

for correct operation; some of the kernel drivers include README files

that include this information.

As explained in the file ide-cd, ATAPI CD-ROMS should be jumpered as

"single" or "master", and not "slave" when only one IDE device is

attached to an interface (although this restriction is no longer

enforced with recent kernels).

4.2.  Configuring and Building the Kernel

When initially installing Linux from CD-ROM you will likely be using a

boot and/or root disk provided as part of a Linux distribution. If

possible, you should choose a boot disk with the kernel driver for

your CD-ROM device type. If you cannot find a boot disk with the

necessary CD-ROM driver, you have several options:

1. Install over a network

2. Boot DOS, and install the Linux files onto your hard disk

3. Boot DOS, and create a set of floppies to install Linux

4. Find someone who can build you a boot disk with the needed CD-ROM

driver

The Linux Installation HOWTO

<http://sunsite.unc.edu/mdw/HOWTO/Installation-HOWTO.html> has more

information on installing Linux. If you purchased Linux on CD-ROM, it

likely also came with some installation instructions (that little

booklet inside the jewel case, and/or files on the CD).

Once Linux has initially been installed, most users will want to

compile their own kernel, usually for one of these reasons:

to support a CD-ROM drive or other hardware

to upgrade to a newer kernel release

to free up memory resources by minimizing the size of the kernel

The Linux Kernel HOWTO <http://sunsite.unc.edu/mdw/HOWTO/Kernel-

HOWTO.html> should be consulted for the details of building a kernel.

I will just mention here some issues that are specific to CD-ROM

drives.

Obviously, you need to compile in support for your CD-ROM drive when

you do a "make config".

If you have an ATAPI CD-ROM drive, you need to answer yes to the

questions:

Enhanced IDE/MFM/RLL disk/cdrom/tape support (CONFIG_BLK_DEV_IDE) [Y/n/?]

Include IDE/ATAPI CDROM support (CONFIG_BLK_DEV_IDECD) [Y/n/?]

For SCSI CD-ROM drives, enable these options:

SCSI support (CONFIG_SCSI) [Y/n/m/?]

SCSI CD-ROM support (CONFIG_BLK_DEV_SR) [Y/n/m/?]

Also enable support for your SCSI host adapter when prompted, e.g.

Adaptec AHA152X support (CONFIG_SCSI_AHA152X) [Y/n/m/?]

For proprietary interface CD-ROM drives, enable the appropriate

driver. You can use the table listed previously to determine the

driver to use for your model.

Virtually all CD-ROMs use the ISO-9660 file system, so you must also

enable:

ISO9660 cdrom filesystem support (CONFIG_ISO9660_FS) [Y/n/m/?]

Although not needed for CD-ROM operation, if you have a sound card

that is supported under Linux you might want to enable and configure

the kernel sound driver at this time as well. The Sound HOWTO

<http://sunsite.unc.edu/mdw/HOWTO/Sound-HOWTO.html> can be a useful

reference here.

You should then follow the usual procedure for building the kernel and

installing it. Don't boot with the new kernel until you create the

device files and set up any boot time parameters as described in the

next section.

The ISO-9660 filesystem and almost all of the CD-ROM drivers can be

built as loadable kernel modules. This scheme allows the kernel

drivers to be loaded and unloaded without rebooting the kernel,

freeing up memory. I recommend you get your CD-ROM installation

running using compiled-in drivers first. How to use modules is

described in the modules documentation and the Kernel HOWTO

<http://sunsite.unc.edu/mdw/HOWTO/Kernel-HOWTO.html>.

If a drive type listed here is not supported by your kernel, you

likely need to upgrade to a newer version.

It is possible that you need to use a driver that is distributed

separately from the kernel source code. This usually involves patching

the kernel. Again, the Kernel HOWTO

<http://sunsite.unc.edu/mdw/HOWTO/Kernel-HOWTO.html> explains how to

do this.

Note that there is a menu-based kernel configuration program invoked

by "make menuconfig" and an X11-based graphical configuration invoked

as "make xconfig". All three configuration methods offer on-line help.

4.3.  Creating Device Files and Setting Boot Time Parameters

The kernel uses device files to identify which device driver to use.

If you are running a standard Linux distribution you may have created

the necessary device files during installation. Under Slackware Linux,

for example, there is a menu-based setup tool that includes CD-ROM

setup, and most systems have a /dev/MAKEDEV script. If you don't use

these methods, you can use the more manual procedure listed in this

section. Even if you use either of these methods, it is recommended

that you at least verify the device files against the information in

this section.

You create the device file by running the shell commands indicated for

your drive type. This should be done as user root. Note that some

Linux distributions may use slightly different CD-ROM device names

from those listed here.

It is recommended that you also create a symbolic link to the CD-ROM

device to make it easier to remember. For example, for an IDE CD-ROM

drive that is the second device on the secondary interface, the link

would be created using

# ln -s /dev/hdd /dev/cdrom

If you want to play audio CDs, you will need to set the protection on

the device file (the real file, not the symbolic link to it) to allow

all users to read, e.g.

# chmod 664 /dev/hdd

# ls -l /dev/hdd

brw-rw-r--   1 root     disk      22,  64 Feb  4  1995 /dev/hdd

When booting Linux, the device drivers attempt to determine whether

the appropriate devices are present, typically by probing specific

addresses. Many of the drivers auto-probe at several addresses, but

because of differences in configuration, possible device conflicts,

and hardware limitations, the drivers sometimes need help identifying

the addresses and other parameters. Most drivers support an option on

the kernel command line to pass this information to the device driver.

This can be done interactively, or more commonly, configured into your

boot loader. With LILO, for example, you would add an append command

such as the following to your /etc/lilo.conf file:

append = "sbpcd=0x230,SoundBlaster"

See the LILO documentation for more information.

In the next section I discuss issues specific to individual device

drivers, including device files, boot parameters, and the capabilities

of the different drivers. You probably only need to read the section

relevant to your drive type. The README files are usually found in the

directory /usr/src/linux/Documentation/cdrom.

4.3.1.  Sbpcd Driver

Principal author: Eberhard Moenkeberg (emoenke@gwdg.de)

Multi-session support: yes (but not all drives)

Multiple drive support: yes

Loadable module support: yes

Reading audio frames: yes (CR-562, CR-563, CD-200 only)

Auto-probing: yes

Device file: /dev/sbpcd, major 25

Configuration file: sbpcd.h

Kernel config option: Matsushita/Panasonic CDROM support?

README file: sbpcd

This driver accepts a kernel command line of the form:

sbpcd=<io-address>,<interface-type>

where the first parameter is the base address of the device (e.g.

0x230), and <interface-type> is one of "SoundBlaster", "LaserMate", or

"SPEA". See the file sbpcd.h for hints on what interface type to use.

Using sbpcd=0 disables auto-probing, disabling the driver.

The device file can be created using:

# mknod /dev/sbpcd b 25 0

Up to four drives per controller are supported. The next three drives

on the first controller would use minor device numbers 1 through 3.

If you have more than one controller, create devices with major

numbers 26, 27, and 28, up to a maximum of 4 controllers (this is 16

CD-ROM drives in total; hopefully enough for most users :-).

See the file sbpcd for more information on this driver.

If you recently bought a CD-ROM drive, don't assume that if it

connects to a SoundBlaster card it should use this kernel driver. Most

CD-ROM drives being sold by Creative Labs are now EIDE/ATAPI drives.

Please don't bug the author with questions about using this driver

with IDE drives. I quote from the README file:

sbpcd really, really is NOT for ANY IDE/ATAPI drive!

Not even if you have an "original" SoundBlaster card with an IDE interface!

So, you better have a look into README.ide if your port address is 0x1F0,

0x170, 0x1E8, 0x168 or similar.

I get tons of mails from IDE/ATAPI drive users - I really can't continue

any more to answer them all. So, if your drive/interface information sheets

mention "IDE" (primary, secondary, tertiary, quaternary) and the DOS driver

invoking line within your CONFIG.SYS is using an address below 0x230:

DON'T ROB MY LAST NERVE - jumper your interface to address 0x170 and IRQ 15

(that is the "secondary IDE" configuration), set your drive to "master" and

use ide-cd as your driver. If you do not have a second IDE hard disk, use the

LILO commands

hdb=noprobe hdc=cdrom

and get lucky.

To make it fully clear to you: if you mail me about IDE/ATAPI drive problems,

my answer is above, and I simply will discard your mail, hoping to stop the

flood and to find time to lead my 12-years old son towards happy computing.

4.3.2.  Sonycdu535 Driver

Principal author: Ken Pizzini (ken@halcyon.com)

Multi-session support: no

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/sonycd535, major 24

Configuration file: sonycd535.h

Kernel config option: Sony CDU535 CDROM support?

README file: sonycd535

This driver accepts a kernel command line of the form:

sonycd535=<io-address>

where <io-address> is the base address of the controller (e.g. 0x320).

Alternatively you can set the address in the file sonycd535.h and

compile it in.

The device file can be created using:

# mknod /dev/sonycd535 b 24 0

Some Linux distributions use /dev/sonycd for this device.  Older

versions of the driver used major device number 21; make sure your

device file is correct.

This driver was previously distributed as a patch but is now part of

the standard kernel. See the file sonycd535 for more information on

this driver.

4.3.3.  Cdu31a Driver

Principal author: Corey Minyard (minyard@-rch.cirr.com)

Multi-session support: yes

Multiple drive support: no

Loadable module support: yes

Reading audio frames: yes

Auto-probing: no

Device file: /dev/cdu31a, major 15

Configuration file: cdu31a.h

Kernel config option: Sony CDU31A/CDU33A CDROM support?

README file: cdu31a

This driver accepts a kernel command line of the form:

cdu31a=<io-address>,<interrupt>,PAS

The first number is the I/O base address of the card (e.g. 0x340). The

second is the interrupt number to use (0 means to use polled i/o). The

optional third parameter should be "PAS" if the drive is connected to

a Pro-Audio Spectrum 16 sound card, otherwise left blank.

If the driver is loaded as a module, it uses a slightly different

format. When loading the driver using the modprobe or insmod command,

the parameters take the form:

cdu31a_port=<io-address> cdu31a_irq=<interrupt>

The base io-address is required while the interrupt number is

optional.

The device file can be created using:

# mknod /dev/cdu31a b 15 0

See the file cdu31a for more information on this driver.

Also see the Web page put together by Jeffrey Oxenreider

(zureal@infinet.com) that covers a lot of common problems with these

drives. It can be found at

<http://www.infinet.com/~zureal/cdu31a.html>.

4.3.4.  Aztcd Driver

Principal author: Werner Zimmermann (zimmerma@rz.fht-esslingen.de)

Multi-session support: yes

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/aztcd0, major 29

Configuration file: aztcd.h

Kernel config option: Aztech/Orchid/Okano/Wearnes (non IDE) CDROM support?

README file: aztcd

This driver accepts a kernel command line of the form:

aztcd=<io-address>

where the parameter is the I/O base address of the card (e.g. 0x340).

The device file can be created using:

# mknod /dev/aztcd0 b 29 0

Note that this driver is for the CDA268-01A only. Other models,

including the CDA268-03I and CDA269-031SE are not proprietary and

should use the IDECD (ATAPI) kernel driver.

See the file aztcd for more information on this driver.

4.3.5.  Gscd Driver

Principal author: Oliver Raupach (raupach@nwfs1.rz.fh-hannover.de)

Multi-session support: no

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/gscd0, major 16

Configuration file: gscd.h

Kernel config option: Goldstar R420 CDROM support?

README file: gscd

This driver accepts a kernel command line of the form:

gscd=<io-address>

specifying the I/O base address of the card (e.g. 0x340).

The device file can be created using:

# mknod /dev/gscd0 b 16 0

See the file gscd and the World-Wide Web site <http://linux.rz.fh-

hannover.de/~raupach> for more information on this driver.

4.3.6.  Mcd Driver

Principal author: Martin  (martin@bdsi.com)

Multi-session support: no

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/mcd, major 23

Configuration file: mcd.h

Kernel config option: Standard Mitsumi CDROM support?

README file: mcd

This is the older driver for Mitsumi drivers that has been available

for some time. You might want to try the newer mcdx driver, which has

some new features but is possibly less stable.

This driver accepts a kernel command line of the form:

mcd=<io-address>,<irq>

specifying the I/O base address of the card (e.g. 0x340) and the IRQ

request number used.

The device file can be created using:

# mknod /dev/mcd b 23 0

See the file mcd for more information on this driver.

4.3.7.  Mcdx Driver

Principal author: Heiko Schlittermann

Multi-session support: yes

Multiple drive support: yes

Loadable module support: yes

Reading audio frames: no (not supported by hardware)

Auto-probing: no

Device file: /dev/mcdx0, major 20

Configuration file: mcdx.h

Kernel config option: Experimental Mitsumi support?

README file: mcdx

This is a newer driver for Mitsumi drivers. The older and possibly

more stable mcd driver is still available.

This driver accepts a kernel command line of the form:

mcdx=<io-address>,<irq>

specifying the I/O base address of the card (e.g. 0x340) and the IRQ

request number used.

The device file can be created using:

# mknod /dev/mcdx0 b 20 0

If you recently bought a Mitsumi CD-ROM drive, don't assume that it

should use this kernel driver. Some Mitsumi models are now EIDE/ATAPI

drives and should use the idecd kernel driver.

See the file mcdx for more information on this driver.

4.3.8.  Cm206 Driver

Principal author: David A. van Leeuwen (david@tm.tno.)

Multi-session support: yes

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: yes

Device file: /dev/cm206cd, major 32

Configuration file: cm206.h

Kernel config option: Philips/LMS CM206 CDROM support?

README file: cm206

The driver accepts a kernel command line of the form:

cm206=<io-address>,<interrupt>

where the first number is the I/O base address of the card (e.g.

0x340). The second is the interrupt channel.

The device file can be created using:

# mknod /dev/cm206cd b 32 0

See the file cm206 for more information on this driver.

4.3.9.  Optcd Driver

Principal author: Leo Spiekman (spiekman@dutette.et.tudelft.nl)

Multi-session support: yes

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/optcd0, major 17

Configuration file: optcd.h

Kernel config option: Experimental Optics Storage ... CDROM support?

README file: optcd

The driver accepts a kernel command line of the form

optcd=<io-address>

to specify the I/O base address of the card (e.g. 0x340).

The device file can be created using:

# mknod /dev/optcd0 b 17 0

See the file optcd for more information on this driver.

4.3.10.  Sjcd Driver

Principal author: Vadim V. Model (vadim@rbrf.msk.su)

Multi-session support: no

Multiple drive support: no

Loadable module support: yes

Reading audio frames: no

Auto-probing: no

Device file: /dev/sjcd, major 18

Configuration file: sjcd.h

Kernel config option: Experimental Sanyo H94A CDROM support?

README file: sjcd

The driver accepts a kernel command line of the form:

sjcd=<io-address>,<interrupt>,<dma>

indicating the base address, interrupt, and DMA channel to be used

(e.g. sjcd=0x340,10,5).

The device file can be created using:

# mknod /dev/sjcd b 18 0

See the file sjcd for more information on this driver.

4.3.11.  SCSI Driver

Principal author: David Giller

Multi-session support: yes (depending on drive)

Multiple drive support: yes

Loadable module support: yes

Reading audio frames: no

Auto-probing: yes

Device file: /dev/scd0, major 11

Configuration file: cdrom.h

Kernel config option: SCSI CDROM support?

README file: none

There are kernel command line option specific to each type of SCSI

controller. See the SCSI HOWTO for more information.

Multiple drives are supported (up to the limit of the maximum number

of devices on the SCSI bus). Create device files with major number 11

and minor numbers starting at zero:

# mknod /dev/scd0 b 11 0

# mknod /dev/scd1 b 11 1

While the kernel driver itself does not support reading digital audio

frames, some SCSI drives have the capability and will work with the

cdda2wav program (which uses the generic SCSI kernel interface).

4.3.12.  IDECD Driver

Principal author: Scott Snyder (snyder@fnald0.fnal.gov)

Multi-session support: yes

Multiple drive support: yes

Loadable module support: no

Reading audio frames: yes (on supported drives)

Auto-probing: yes

Device file: /dev/hd{a,b,c,d},  major 22

Configuration file: cdrom.h

Kernel config option: Include support for IDE/ATAPI CDROMs?

README file: ide-cd

This is the driver for ATAPI CD-ROMS. The driver accepts a kernel

command line of the form

hdx=cyls,heads,sects,wpcom,irq

or

hdx=cdrom

where hdx can be any of {hda,hdb,hdc,hdd}, or simply hd, for the

"next" drive in sequence. Only the first three parameters are required

(cyls,heads,sects). For example hdc=1050,32,64 hdd=cdrom.

Getting the IDE driver to recognize your CD-ROM drive can be tricky,

especially if you have more than 2 devices or more than one IDE

controller. Usually all that is required is to pass the right command

line options from LILO. The file  </usr/src/linux/Documentation/ide-

cd> explains how to do this. Read it carefully.

Recent Linux kernels have better support for multiple IDE devices. If

you have problems with an older kernel, upgrading may help.

Some IDE controllers have hardware problems which the kernel driver

can work around. You may need to pass additional parameters to the

driver to enable this. See the documentation for details.

4.4.  Booting the Linux Kernel

You can now reboot with the new kernel. Watch for a message such as

the following indicating that the CD-ROM has been found by the device

driver (the message will vary depending on the drive type):

hdd: NEC CD-ROM DRIVE:282, ATAPI CDROM drive

If the bootup messages scroll by too quickly to read, you should be

able to retrieve them using dmesg or tail /var/adm/messages.

If the drive is not found, then a problem has occurred, See the

section on troubleshooting.

4.5.  Mounting, Unmounting, and Ejecting Devices

To mount a CD-ROM, insert a disc in the drive, and run the mount

command as root (this assumes you created a symbolic link to your

device file as recommended above and that an empty directory

/mnt/cdrom exists):

# mount -t iso9660 -r /dev/cdrom /mnt/cdrom

The CD can now be accessed under the directory /mnt/cdrom.

There are other options to the mount command that you may wish to use;

see the mount(8) man page for details.

You can add an entry to /etc/fstab to automatically mount a CD-ROM

when Linux boots or to specify parameters to use when it is mounted;

see the fstab(5) man page.

Note that to play audio CDs you should not try to mount them.

To unmount a CD-ROM, use the umount command as root:

# umount /mnt/cdrom

The disc can only be unmounted if no processes are currently accessing

the drive (including having their default directory set to the mounted

drive). You can then eject the disc. Most drives have an eject button;

there is also a standalone eject program that allows ejecting CD-ROMs

under software control.

Note that you should not eject a disc while it is mounted (this may or

may not be possible depending on the type of drive). Some CD-ROM

drivers can automatically eject a CD-ROM when it is unmounted and

insert the CD tray when a disc is mounted (you can turn this feature

off when compiling the kernel or by using a software command).

Its possible that after playing an audio CD you may not be able to

mount a CD-ROM. You need to send a CD audio "stop" command (using a CD

player program) before trying the mount. This problem only appears to

occur with the SBPCD driver.

Stephen Tweedie (sct@dcs.ed.ac.uk) has written the Supermount package

which provides transparent mounting of removable media including CD-

ROM. You can find it at

<ftp://sunsite.unc.edu/pub/linux/patches/diskdrives/>.

4.6.  Troubleshooting

If you still encounter problems after following the instructions in

the HOWTO, here are some things to check. The checks are listed in

increasing order of complexity. If a check fails, solve the problem

before moving to the next stage.

4.6.1.  Step 1: Make sure you are really running the kernel you com

piled

You can check the date stamp on the kernel to see if you are running

the one that you compiled with CD-ROM support. You can do this with

the uname command:

% uname -a

Linux fizzbin 2.0.18 #1 Fri Sep 6 10:10:54 EDT 1996 i586

or by displaying the file /proc/version:

% cat /proc/version

Linux version 2.0.18 (root@fizzbin) (gcc version 2.7.2) #1 Fri Sep 6 10:10:54 EDT 1996

If the date stamp doesn't seem to match when you compiled the kernel,

then you are running an old kernel. Did you really reboot? If you use

LILO, did you re-install it (typically by running /sbin/lilo)? If

booting from floppy, did you create a new boot floppy and use it when

booting?

4.6.2.  Step 2: Make sure the proper kernel drivers are compiled in

You can see what drivers are compiled in by looking at /proc/devices:

% cat /proc/devices

Character devices:

1 mem

2 pty

3 ttyp

4 ttyS

5 cua

7 vcs

Block devices:

3 ide0

22 ide1

First look for your CD-ROM device driver. These are all block devices,

in this case we can see that the idecd driver with major number 22 was

present.

Also make sure that ISO-9660 filesystem support was compiled in, by

looking at /proc/filesystems:

% cat /proc/filesystems

ext2

msdos

nodev   proc

iso9660

You can also see what i/o port addresses are being used by a driver

with the file /proc/ioports:

howto % cat /proc/ioports

...

0230-0233 : sbpcd

...

If any of the drivers you thought you compiled in are not displayed,

then something went wrong with the kernel configuration or build.

Start the installation process again, beginning with configuration and

building of the kernel.

4.6.3.  Step 3: Did the kernel detect your drive during booting?

Make sure that the CD-ROM device was detected when the kernel booted.

You should have seen a message on bootup. If the messages scrolled off

the screen, you can usually recall them using the dmesg command:

% dmesg

or

% tail /var/adm/messages

If your drive was not found then something is wrong. Make sure it is

powered on and all cables are connected. If your drive has hardware

jumpers for addressing, check that they are set correctly (e.g. drive

0 if you have only one drive). ATAPI CD-ROMS must be jumpered as

"single" or "master", and not "slave" when only one IDE device is

attached to an interface. If the drive works under DOS then you can be

reasonably confident that the hardware is working.

Many kernel drivers using auto-probing, but some do not, and in any

case the probing is not always reliable. Use the kernel command line

option listed for your kernel driver type. You may want to try several

different values if you are not sure of the i/o address or other

parameters. LILO can be (and usually is) configured to allow you to

enter the parameters manually when booting.

Another possibility is that you used the wrong kernel driver for your

CD-ROM driver. Some documentation may refer to proprietary interfaces

as IDE, leading some to mistakenly believe they are ATAPI drives.

Another possibility is that your drive (or interface card) is one of

the "compatible" type that requires initialization by the DOS driver.

Try booting DOS and loading the vendor supplied DOS device driver.

Then soft boot Linux using Control-Alt-Delete.

If your drive is not listed in this document, it is possible that

there are no drivers for it available under Linux. You can check with

some of the references listed at the end of this document for

assistance.

4.6.4.  Step 4: Can you read data from the drive?

Try reading from the CD-ROM drive. Typing the following command should

cause the drive activity light (if present) to come on and no errors

should be reported. Use whatever device file is appropriate for your

drive and make sure a CD-ROM is inserted; use Control-C to exit.

# dd if=/dev/cdrom of=/dev/null bs=2048

^C

124+0 records in

124+0 records out

If this works, then the kernel is communicating with the drive and you

can move on to step 5.

If not, then a possible cause is the device file. Make sure than the

device file in the /dev directory has the correct major and minor

numbers as listed previously for your drive type. Check that the

permissions on the dev